Eagle Vision Specification

Video Inputs

- Composite video connector (BNC)

- S-Video color input connector (4-pin DIN)

- Two RCA connectors

Video Decoder

- NTSC & Pal (RGB, Composite, S-video, YUV)

- 24/16/15/8 bit

- NTSC 640*480(12.27MHz), PAL 786*576(14.75MHz)

- Digital control of brightness, contrast, hue, and saturation

Video Format

- 24/16/15 bit RGB or YUV 4:2:2

- Y8 monochrome

RGB Video Pixel Format

- 888-16.7 million colors

- 565-65 thousand colors

- 555.32 thousand colors

- 8-256 level monochrome

Video Throughput Performance

- Full size, full speed video delivery to and from system or VGA memory

- Full bandwidth PCI Bus master read and write (up to 132MB/sec)

Video Output Display

- NTSC 640*480 (PAL 768*576)

- 30 frames/ sec, 60 fields/sec, or 30 fields/sec

Video Input Cable

- BNC, S-Video, RS170 cable